Risk Assessment

RISK RATING = LIKELIHOOD* CONSEQUENCES

| A likelihood of occurrence |  | B Consequences (Injury severity) |  |
| --- | --- | --- | --- |
| Almost certain | 5 | Extreme | 5 |
| Likely | 4 | Major | 4 |
| Possible | 3 | Moderate | 3 |
| Unlikely | 2 | Minor | 2 |
| Rare/Remote | 1 | Negligible | 1 |

| Risk classification | Risk value |
| --- | --- |
| High | 16-25 |
| Medium | 9-15 |
| Low | 1-8 |

| 5*5 Risk Matrix |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |
|  | 1 | 2 | 3 | 4 | 5 |

| HIGH | The risk should ceased immediately until further control measures to mitigate the risk are introduced. |
| --- | --- |
| MEDIUM | The risk should only be tolerated for short term and then only whilst further control measures to mitigate the risk are being planned and introduced, within a defined time period. |
| LOW | Are largely acceptable, subject to reviews periodically, or after significant change etc… |

| Project Name | Potential Hazard | Possible Result (Injury/ Damage/ Loss) | Risk Evaluation |  |  | SWP (Safe work procedure) | Control (Supervision/ register) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | R |  |  |

| - Working on Unprotected Slab edges Unprotected slab edge | - Falling from heights OSHA’s fall protection standard: Each employee on working surface between 6 and 15 feet from the edge, you will need standard fall protection, including Guardrail and fall arrest. | - Fatal injury (Including death) | 20 = HIGH RISK == 4 | 5 | 20 | - Maintain Guardrail to protect workers from falling down. - Provide strong barricades to close slab peripheries. - Provide Toolbox briefings. - Post warning signs and use warning tapes. | - Site safety representative (Site Engineer) - Forman - EHS Engineer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Working on Unprotected Staircases Unprotected stair edge | - Falling from heights The 6-foot rule: Workers requires the use of fall protection when construction workers are working at heights of 6 feet or greater above a lower level. | - Fatal injury (Including death | 12 = MID RISK == 3 | 4 | 12 | - Maintain Guardrail to protect workers from falling down. - Provide strong barricades to close slab peripheries. - Provide Toolbox briefings. - Post warning signs and use warning tapes. | - Site safety representative (Site Engineer) - Forman - EHS Engineer |
| - General Housekeeping Messy working areas | - Slip - Trip - Fall hazards and - Material damage OSHA’s crane standard: All places of employment, passageways, storerooms, service rooms, and walking-working surfaces are kept in a clean, orderly, and sanitary condition. | - Abrasion, and bleeding - Property damage. | 6 = LOW RISK == 3 | 2 | 6 | - All passageways, gangways and access ways shall be kept free of materials, supplies, and obstructions at all times. - Tools, materials, extension cords, hoses, or debris shall not cause tripping or other hazards. - Material shall stack properly. - Avoid corrections and damage to the property. | - Site safety representative (Site Engineer) - Forman - EHS Engineer |